NORTHERN TERRITORY OF AUSTRALIA

BUILDING REGULATIONS

As in force at 1 January 2012

Table of provisions

Part 1	Preliminary

1	Citation	1

2	Interpretation	1

3	Exemptions from regulations and sections 55 and 65 of the Act	3

3A	Prescribing classes of buildings for and exemptions from section 61 of the Act	4

3B	Exemption from section 62 of the Act	4

Part 2	Building standards

4	Building Code, &c., adopted	5

5	Accrediting persons or bodies	5

6	Application for building permit	6

7	Time for the supply of additional information	6

8	Reporting authority	6

9	Limitation of time for reporting authority	7

10	Limitation of time for building certifier to decide application for building permit	7

11	Prescribed approvals for the purposes of section 59(1)(b) and (c)	7

12	Historic buildings	8

13	Duration of building permit	8

14	Distribution of plans and drawings	9

15	Access to records	9

Part 3	Inspections, building permits and occupancy permits

Division 1	Inspections

15A	Inspection stages	9

16	Drainage works to be notified	10

Division 2	Building permits

16A	Documents relating to building permits to be provided to Director	10

Division 3	Occupancy permits

17	Limitation of time for reporting authorities in relation to occupancy permit	11

18	Permits not required for buildings in certain areas	11

18A	Documents relating to occupancy permits to be provided to Director	11

18B	Consolidation of occupancy permits	12

18C	Documents to accompany application for occupancy permit	12

19	Limitation of time for building certifier to decide application	13

20	Change of use	13

Part 3A	Declarations relating to particular building work

20A	Application of Part	14

20B	Declaration for building work requiring occupancy permit	14

20C	Declaration for building work not requiring occupancy permit	15

20D	Requirements for declaration under regulation 20C	15

20E	Declaration by corporation after prescribed event	16

20F	Building certifier to give copy of declaration to Director	17

Part 4	Approvals etc., by other persons or bodies

21	Certificates by other persons	17

Part 5	Protection of adjoining property

22	Protection of adjoining property	18

Part 6	Enforcement of building standards

23	Forms of building notice, building order	18

24	Management and fire evacuation training	18

25	Notification, cancellation or amendment of building order	19

Part 7	Appeals

26	Enforcement of determination	19

Part 8	General enforcement provision

27	Recovery of penalties	19

28	Orders to be enforced by Local Court	19

29	Notice of authorisation	19

Part 9	Siting requirements

30	Interpretation	20

31	Application	20

32	Minimum distances for Class 1 or Class 10 buildings	20

33	Exceptions	20

34	Minimum distance for a Class 1 building in Municipality of Palmerston	21

35	More than one Class 1 building on one allotment	21

36	Plot ratio of Class 1 buildings	21

Part 9A	Relocation of approved Class 1a Buildings to approved rural land

36A	Definitions	22

36B	Application of Part	22

36C	Approved Class 1a Building	23

36D	Footings and fixture work	23

36E	Plumbing connection work	24

36F	Electrical installation connection work	24

36G	Occupancy permit and structural report	24

Part 10	Areas liable to flooding

37	Flood prone areas	25

38	Flood levels	25

39	Requirements in flood prone areas	25

Part 11	Building practitioners

39A	Sub-categories of building practitioner	26

40	Condition for registration: professional indemnity insurance	26

40A	Condition of registration: HBCF plumber policy	27

40B	Last day for application for renewal of registration	27

41	Fees	27

Part 11A	Building contractors

Division 1	Prescribed matters for section 48A of the Act

41A	Building work	27

41B	Building contractors for detached houses and attached dwellings and Class 2 buildings of not more than 2 storeys	28

41C	Building contractors for attached dwellings and Class 2 buildings of more than 2 storeys	29

41D	Supervision by building contractor	30

41E	Minimum value of building work	30

Division 2	Prescribed matters for section 48B of the Act

41F	Building contractors	30

41G	Building work	30

41H	Contents of building contracts	31

41J	Minimum value of building work	32

Division 3	Prescribed matters for section 48C of the Act

41K	Building contractors	32

41L	Building work	32

Part 11B	Owner-builders

Division 1	Prescribed matters for section 48D of the Act

41N	Building work	33

41P	Minimum value of building work	34

41Q	Exemption for certain building contractors	34

Division 2	Prescribed matters for section 48F of the Act

41R	Period required to elapse	34

Part 12	Miscellaneous

42	Offences	34

43	Transitional	34

Part 13	Matters for commencement of Building Amendment Act 2004

44	Definitions	35

45	Inspection stages	35

46	Exemption of work from sections 63 and 70	35

47	Exemption of occupancy permit for building from section 72	36

Part 14	Transitional matters for Parts 4A and 4B of Act

48	Registration of building contractors	36

49	Building permits granted before commencement of Parts 4A and 4B of the Act	36

Part 15	Transitional matters for inspection stages of residential building work

50	Inspection stages for pre-July 2006 residential building work	36

Part 16	Transitional matters for Building Legislation Amendment Act 2010

51	Definitions	37

52	Declaration made by corporation after commencement day for building work completed before commencement day	37

Schedule 1	Prescribed fees

Schedule 2

Schedule 3

Schedule 4	Flood prone areas

Schedule 5	Modifications to the National plumbing code

ENDNOTES

northern territory of australia

____________________

This reprint shows the Regulations as in force at 1 January 2012.  Any amendments that commence after that date are not included.

____________________

BUILDING regulations

Regulations under the Building Act

Part 1	Preliminary

1	Citation

These regulations may be cited as the Building Regulations.

2	Interpretation

(1)	In these regulations:

approved Class 1a Building, see regulation 36C(1).

approved rural land, see regulation 36A.

building contractor residential (restricted) means a building practitioner registered in the category of building contractor residential (restricted).

building contractor residential (unrestricted) means a building practitioner registered in the category of building contractor residential (unrestricted).

Class 2 building, for regulation 15A and Part 11A, includes:

(a)	a building consisting only of one or more storeys that may be classified under the Building Code as a Class 2 building because of Part A3.3(a)(i) of the Code; and

(b)	a building consisting only of:

(i)	one or more storeys that are Class 2 buildings; and

(ii)	one or more storeys that are Class 7a buildings for the sole use of the residents of, and visitors to, the Class 2 buildings.

completed building work, for Part 3A, means the building work specified in regulation 20A(1) and completed as specified in regulation 20A(2).

drawings, of a building, include a site plan and floor plan of, and elevations for, the building.

flood prone area means a part of Territory specified in Schedule 4.

original building status, of an approved Class 1a Building, see regulation 36C(1)(e).

original site, of an approved Class 1a Building, see regulation 36A.

pergola does not include a building that has a roof or sides that are impermeable to the wind.

planning scheme, see section 3(1) of the Planning Act.

post-relocation building work, for an approved Class 1a Building, see regulation 36A.

relevant building law, in relation to a building, means the law (including codes and standards under the law) in force in the Territory that, at the time of the erection of the building, regulated the building work carried out for the erection.

the Building Code means the Building Code of Australia published by or on behalf of the Australian Building Codes Board as amended from time to time.

the National Plumbing Code means the National Plumbing and Drainage Code set out in AS3500 published by the Standards Association of Australia as amended from time to time.

(2)	For the purposes of these regulations the erection of a building that has been removed and transported from another site or another location on the same site is to be regarded as the erection of a building.

(2A)	However, subregulation (2) does not apply to the erection of an approved Class 1a Building on approved rural land.

(3)	A reference in these regulations to a building as a building of a particular Class is a reference to a building of that Class as specified under the classification in Part A3.2 of the Building Code.

(4)	In reckoning time for the purposes of these regulations public holidays, Saturdays and Sundays shall be excluded.

3	Exemptions from regulations and sections 55 and 65 of the Act

(1)	These regulations do not apply to:

(a)	temporary offices and sheds used by builders on or about the site of any building on which building work is being carried out or used by contractors in carrying out works for any public authority on or about the site of the work and used exclusively for the purposes of that building or work;

(b)	any building classified as a Class 10 Building to which electrical, plumbing or drainage services are not provided and that is:

(i)	situated not less than 40 m from any boundary of the allotment on which it stands or is to be constructed, has an aggregate roof area not exceeding 12 m2 and is not more than 2100 mm in height above the mean natural ground level of its site;

(ii)	situated not less than 70 m from any boundary of the allotment on which it stands or is to be constructed, has an aggregate roof area not exceeding 30 m2 and is not more than 2400 mm in height above the mean natural ground level of its site;

(c)	a fence that offers minimal resistance to wind loading such as a chain mesh or a similar kind of fence and in any event a fence that does not exceed a height of 1m;

(d)	a pergola that has an area not exceeding 30 m² and that is not less than 50 mm from a building to which these Regulations apply and that is not attached or connected in any way to any building; or

(e)	minor plumbing maintenance work such as the replacement of a component by a similar component but not including drainage work.

(2)	Sections 55 and 65 of the Act do not apply to or in relation to:

(a)	any building classified as a Class 10 Building that is of a type approved by the Director and is constructed in accordance with that approval;

(b)	plumbing or drainage work that is not done in connection with or incidentally to other building work if:

(i)	notice of the work undertaken is given to the Director not later than 7 days after the work is carried out; and

(ii)	the notice is accompanied by a certificate in the approved form of the relevant building practitioner that the work conforms to the relevant code or standard.

3A	Prescribing classes of buildings for and exemptions from section 61 of the Act

(1)	For section 61 of the Act, the following are prescribed classes of buildings:

(a)	Class 1 Buildings;

(b)	Class 2 Buildings that have no more than 3 residential storeys;

(c)	Class 10 Buildings.

(2)	Despite subregulation (1)(a), section 61 of the Act does not apply in relation to a Class 1 Building if:

(a)	the building is an approved Class 1a Building; and

(b)	the building work to be carried out on the building is in connection with:

(i)	the removal of the building from its current site for relocation on approved rural land; or

(ii)	post-relocation building work.

(3)	To avoid doubt:

(a)	a person is not required to hold a policy of insurance mentioned in section 61 of the Act in relation to building work to be carried out on:

(i)	a building that is not of a prescribed class of buildings mentioned in subregulation (1); or

(ii)	a Class 1 Building mentioned in subregulation (2); and

(b)	building work to be carried out on a building mentioned in paragraph (a)(i) or (ii) must not be covered by an insurance policy mentioned in section 61 of the Act.

(4)	Section 61 of the Act does not apply to a public authority.

3B	Exemption from section 62 of the Act

Section 62 of the Act does not apply to a public authority.

Part 2	Building standards

4	Building Code, &c., adopted

(1)	Subject to these Regulations, the Building Code, the National Plumbing and Drainage Code as modified in Schedule 5 and the Code of Practice for Small On-site Sewage and Sullage Treatment Systems and the Disposal or Reuse of Sewage Effluent, November 1996, published by Territory Health Services apply to any building that can be classified according to use under Part A3.2 of the Building Code and to any building work referred to in the Act or in the Building Code.

(2)	Building work shall conform with these regulations but where the Director is of the opinion that any building work consists only of minor work and does not adversely affect the safety of persons accommodated in or resorting to a building or property in or in the vicinity of a building the Director may determine that these Regulations, or such of the provisions of these Regulations as are specified by the Director, do not or does not apply, as the case requires, in relation to such work and any such determination has effect accordingly.

5	Accrediting persons or bodies

(1)	The following persons or bodies are prescribed as accrediting persons or bodies for the purposes of section 53 of the Act:

(a)	the Australian Building Codes Board;

(aa)	a person or body duly authorised under the CodeMark scheme administered by the Australian Building Codes Board;

(b)	the Building Advisory Committee established under section 9 of the Act;

(c)	the Standards Association of Australia;

(d)	the Power and Water Corporation established by the Power and Water Corporation Act.

(2)	A person or body referred to in subregulation (1) may issue certificates of accreditation in relation to building products, construction methods, designs, components and systems accredited by the person or body.

(3)	A certificate of accreditation issued under subregulation (2) shall be in the form approved by the Director.

6	Application for building permit

An application for a building permit shall be accompanied by:

(a)	drawings showing the plan and proposed usage at each floor level, elevations, sections and dimensions of the building, the sizes and locations of structural members to a scale of not less than 1:100;

(b)	drawings containing sufficient detail and at a scale appropriate to the work carried out or to be carried out to show the plumbing and drainage work to be carried out;

(c)	drawings to a scale of not less than 1:500 showing:

(i)	the boundaries and dimensions of the allotment, relevant easements and adjacent streets;

(ii)	the position and dimension of the building and the relationship of the building to the boundaries of the allotment, existing buildings on the allotment and adjoining allotments together with details of the purposes for which the buildings are to be used; and

(iii)	the levels of the site and of the floors of the building in relation to an adjoining street channel, if any; and

(d)	in the case of an alteration or modification of a building – a statement that describes the purposes for which the building has been used and is to be used.

7	Time for the supply of additional information

The time within which additional information is required to be supplied by an applicant for a building permit is the time specified by the building certifier or 40 days, whichever is the greater.

8	Reporting authority

(1)	A person or body specified in column 1 of Schedule 2:

(a)	is a reporting authority in relation to the class of buildings or building work, as the case may be, specified opposite to that person or body in Column 2 of that Schedule; and

(b)	is to report upon such of the matters specified opposite to that class of building or building work, as the case requires, in Column 3 of that Schedule as are relevant in relation to the buildings or building work the subject of a request for a report or consent.

(2)	Where a report is required from a reporting authority the building certifier shall supply the reporting authority with sufficient detailed drawings and other information to enable the reporting authority to properly consider the matter.

(3)	Nothing in subregulation (1) affects any consent or approval required to be given in relation to a building or building work under any other Act or regulation under any other Act.

9	Limitation of time for reporting authority

A reporting authority is taken to have supplied a report in relation to an application for a building permit if the report is not supplied within 10 days of the receipt of the request for the report at the head office of the reporting authority in the Territory as determined by the Director.

10	Limitation of time for building certifier to decide application for building permit

The time within which a building certifier shall decide an application for a building permit is:

(a)	if additional information is not required by the building certifier under section 58 of the Act – 20 days from the day that a completed application is made to the building certifier;

(b)	if additional information is required by the building certifier under section 58 of the Act – 20 days from the day that all the additional information required by the building certifier is supplied to the building certifier.

11	Prescribed approvals for the purposes of section 59(1)(b) and (c)

(1)	The following consents and approvals are prescribed for the purposes of section 59(1)(b) of the Act:

(a)	if the proposed building is to be constructed on Crown land or the building work is proposed to be carried out on Crown land, the approval of the person or body administering the land;

(b)	the approval of any person or body having a right under law to an easement over the land on which or adjacent to which the building is to be constructed or building work is to be carried out; and

(c)	if the building work is to be undertaken in relation to a building listed as a heritage building or involves any other thing listed under the Heritage Conservation Act, the consent of the Minister to whom the administration of the Heritage Conservation Act is allotted by the Administrator.

(2)	The following conditions are prescribed for the purposes of section 59(1)(c) of the Act:

(a)	any condition imposed on a permit in force under the Planning Act;

(c)	any condition imposed by a covenant or encumbrance on the title of the land on which the building work is to be carried out.

12	Historic buildings

The person specified as the person for the purpose of section 60 of the Act in relation to historic buildings is the Minister to whom the administration of the Heritage Conservation Act is allotted by the Administrator.

13	Duration of building permit

(1)	A building permit is valid for a period of 2 years commencing from the date on which it is granted.

(2)	The time referred to in subregulation (1) may be extended if an application for that purpose is made to the building certifier before the expiration of the time for which the building permit is valid.

(3)	Any extension of time granted under subregulation (2) shall be notified to the Director and the notification must include the reasons for which the extension is granted.

(4)	If a building permit lapses solely because of a prescribed event, and the person granted the permit makes an application under section 73C of the Act, the time referred to in subregulation (1) is taken to have been extended for a period decided by the Director and notified to the applicant during consideration of the application.

14	Distribution of plans and drawings

Where a building permit has been granted a copy of the application, approved drawings and certificates which the certifier has relied upon for the purposes of section 40 of the Act and building permit shall be forwarded to:

(a)	the Director; and

(b)	the owner or the owner's agent.

15	Access to records

The owner, or the agent of the owner, of a building in relation to which a building permit has been granted under the Act may:

(a)	inspect a copy of the drawings and documents referred to in regulation 14 at the office of the person or body that has approved the application or at the office of the Director; and

(b)	request the person or body by whom any application has been granted to stamp or endorse one or more copies of the drawings and documents as evidence of that approval and any such request shall be complied with on payment of a reasonable fee.

Part 3	Inspections, building permits and occupancy permits

Division 1	Inspections

15A	Inspection stages

(1)	For section 62(2) of the Act, the inspection stages for residential building work are those stages named and described in the table below that are relevant to the work.

|  | Inspection Stage | Description |
| --- | --- | --- |
| 1. | pre-pour | before pouring the footings, ground floor slab or other in situ concrete building element |
| 2. | frame | before covering the framework for floors, walls, roof or other building element |
| 3. | block wall | before pouring any reinforced masonry or block walls |
| 4. | fire separation | before covering walls, floors or ceilings, for the purpose of checking that fire resistance levels comply with the Building Code (see note) |
| 5. | wet area | before covering waterproofing in wet areas |
| 6. | final | after completing the building work but before issuing an occupancy permit in relation to the work |

Note for table

A fire separation inspection may form part of an inspection of the building work carried out at one or more of the other inspection stages.

(2)	For subregulation (1), residential building work is work for or in connection with the construction of any of the following:

(a)	a Class 1a building;

(b)	a Class 2 building;

(c)	a Class 10 building.

16	Drainage works to be notified

The plumber or drainer carrying out drainage works shall:

(a)	notify the Director of the completion of drainage work prior to covering up those works; and

(b)	stop carrying out those works if directed to do so by the Director.

Division 2	Building permits

16A	Documents relating to building permits to be provided to Director

For section 42(1)(c) of the Act, copies of the following documents must accompany the copy of the building permit:

(a)	any drawings of the building work to be carried out that form part of the permit;

(b)	a schedule of the inspections the building certifier proposes to carry out under section 63 of the Act, including at the inspection stages prescribed by regulation 15A at which the work must be inspected under section 62 of the Act;

(c)	any certificates issued by building practitioners in relation to the work on which the building certifier has relied under section 40 of the Act;

(d)	evidence of any contract required under section 48B of the Act;

(e)	any owner-builder certificate required under section 48D of the Act;

(f)	the evidence provided to the building certifier under section 59(1B) of the Act of any policy of insurance required under Part 4C;

(g)	any relevant planning consents and approvals referred to in section 59(1)(b) of the Act;

(h)	any other approvals, certificates or other documents relied on by the building certifier in issuing the permit.

Division 3	Occupancy permits

17	Limitation of time for reporting authorities in relation to occupancy permit

A reporting authority is taken to have supplied a report in relation to an application for an occupancy permit if the report is not supplied within 10 days of the date of the receipt of the request for the report at the head office of the reporting authority in the Territory as determined by the Director.

18	Permits not required for buildings in certain areas

(1)	For section 65(1) of the Act, an occupancy permit is not required for a building the subject of a declaration in force under subregulation (2).

(2)	The Minister may, by notice in the Gazette, declare that an occupancy permit is not required for a specified building in a specified part of the Territory.

18A	Documents relating to occupancy permits to be provided to Director

For section 42(1)(c) of the Act, copies of the following documents must accompany the copy of the occupancy permit in respect of building work:

(a)	the declaration required to accompany the application for the permit under section 69 of the Act;

(b)	all the documents required to accompany the application for the permit under section 69A(1)(a) to (c) and (e);

(c)	evidence of compliance with any relevant planning consents and approvals referred to in section 69A(1)(d) of the Act;

(d)	any other approvals or other documents relied on by the building certifier in issuing the permit that the certifier has not already provided under regulation 16A(h).

18B	Consolidation of occupancy permits

(1)	Subregulation (2) applies if 2 or more occupancy permits have been granted in relation to a building.

(2)	A building certifier:

(a)	may issue, in place of the occupancy permits, a consolidated occupancy permit for the whole building; and

(b)	if the building certifier does so – must revoke the permits being replaced.

18C	Documents to accompany application for occupancy permit

(1)	For section 69A(1)(e) of the Act, the prescribed documents are the certificates or other documents relied on by the building certifier in issuing the occupancy permit that relate to the manufacture and assembly off-site, or installation on-site, of any components of the building work by a person other than the person required to make the declaration referred to in regulation 18A(a).

(2)	For subregulation (1), components of building work include the following:

(a)	termite management systems;

(b)	prefabricated roof trusses;

(c)	wet area sealing products;

(d)	prefabricated window and door frames;

(e)	glazing;

(f)	fire safety products or installations, including fire doors, smoke alarms, sprinklers and sprinkler systems, fire hose reels and sealing of penetrations.

Examples of certificates for regulation 18C

1.	A manufacturer's certificate for roof trusses.

2.	A wet areas certificate.

3.	A glazing certificate.

19	Limitation of time for building certifier to decide application

The period within which a building certifier shall decide an application for an occupancy permit is 20 days commencing from the day on which the application for the occupancy permit is lodged with the building certifier.

20	Change of use

(1)	This regulation applies to any building, whether erected before or after the commencement of these Regulations.

(2)	The use of a building shall not be changed to another use unless the change to that other use has been approved by a building certifier.

(3)	Before making any change in the use of a building the owner of the building or the owner's agent shall apply in writing to a building certifier setting out the nature of the proposed other use.

(4)	A building certifier shall not approve a change of use of a building to another use unless:

(a)	the building will conform with the requirements of the Building Code applicable to that other use;

(b)	all relevant planning or other consents, reports or approvals (if any) required under the Act and these regulations have been obtained or supplied;

(c)	all planning or other conditions, if any, have been complied with.

(5)	Upon approving of a change of use the building certifier shall:

(a)	revoke the existing occupancy permit issued in relation to the building (if any);

(b)	prepare an occupancy permit in accordance with the Act and these regulations with respect to the new use;

(c)	issue the original of the occupancy permit to the person proposing the change of use; and

(d)	forward a copy of the occupancy permit to the Director.

Part 3A	Declarations relating to particular building work

20A	Application of Part

(1)	This Part applies in relation to all building work carried out by a person under a building permit, including building work for a building mentioned in regulation 18, except:

(a)	prescribed building work to which section 48C of the Act applies; and

(b)	building work to which section 48G of the Act applies.

(2)	For this Part, a person completes building work under a building permit when the person completely ceases to carry out the building work specified in the building permit, regardless of whether the work is fully or partially completed.

20B	Declaration for building work requiring occupancy permit

(1)	This regulation applies in relation to all completed building work that requires an occupancy permit.

(2)	The person who has completed the building work must make a declaration mentioned in section 69(1) of the Act and in accordance with section 69(2) of the Act.

(3)	The person must make the declaration not later than 14 days after completing building work for a Class 1a, Class 2 or Class 10 building, unless regulation 18 applies to the building.

Penalty:	$10 000.

(4)	The person who makes a declaration as required by subregulation (3) must, not later than 14 days after completing the building work, give the declaration to the building certifier who granted the building permit for the work.

Penalty:	$1 000.

Note for subregulations (2), (3) and (4)

In relation to Class 3 to Class 9 buildings, subregulation (2) applies but no time limit is imposed under subregulation (3) or (4).

(5)	If the person who completed the building work was acting for a corporation and ceased to act after completing the work, the person is not excused from making a declaration under subregulation (2) or (3) relating to the work he or she completed.

20C	Declaration for building work not requiring occupancy permit

(1)	This regulation applies in relation to completed building work for a Class 1a or Class 10 building to which regulation 18 applies.

(2)	The person who has completed the building work must, not later than 14 days after completing the work, make a declaration in accordance with regulation 20D(1) and (2).

Penalty:	$10 000.

(3)	The person must, not later than 14 days after completing the building work, give the declaration to the building certifier who granted the building permit for the work.

Penalty:	$1 000.

(4)	If the person who completed the building work was acting for a corporation and ceased to do so after completing the work, the person is not excused from making a declaration under subregulation (2) relating to the work he or she completed.

20D	Requirements for declaration under regulation 20C

(1)	A declaration made for regulation 20C must be in an approved form and include a statement that the building work to which it relates has been carried out in accordance with the building permit granted for the work.

(2)	The declaration must be made by the natural person who carried out the building work or was in charge of carrying out the work.

(3)	However, subregulation (2) does not apply if the declaration is made for a corporation under regulation 20E.

(4)	A person must not make a false statement in a declaration mentioned in subregulation (1).

Penalty:	$10 000.

(5)	A person who makes a declaration for a corporation under regulation 20E must not be prosecuted for an offence against subregulation (4) if the person made the declaration in good faith on the basis of information given to the person by the corporation.

(6)	Subregulation (5) does not prevent the corporation, or a director of the corporation, from being prosecuted for an offence against subregulation (4).

20E	Declaration by corporation after prescribed event

(1)	This regulation applies in relation to completed building work mentioned in regulation 20B or 20C if all of the following circumstances apply:

(a)	the building work has been completed by a person acting for a corporation (the responsible builder);

(b)	the responsible builder has failed to make a declaration under the regulation solely because of a prescribed event;

(c)	the corporation is satisfied the building work was carried out by the responsible builder in accordance with the building permit granted for the work.

(2)	For subregulation (1)(c), the corporation must:

(a)	take all the steps necessary, within a reasonable time and using proper care and diligence, which may include engaging suitably qualified and experienced persons to inspect and give reports about the building work; and

(b)	ensure other prescribed certification required for the building work carried out by the responsible builder has been obtained.

(3)	As soon as practicable after the corporation has satisfied itself for subregulation (1)(c), it must:

(a)	give the information it has relied on to an appropriate person; and

(b)	ensure that a declaration is made under subregulation (4) and given to a building certifier as required by that subregulation.

Penalty:	$50 000.

(4)	Not later than 14 days after the appropriate person has been given the information under subregulation (3), the person must:

(a)	make a declaration mentioned in section 69(1) of the Act or as required by regulation 20D(1), as applicable, relating to the building work completed by the responsible builder; and

(b)	include in the declaration details about:

(i)	the prescribed event; and

(ii)	the way in which the corporation has satisfied itself under subregulation (2); and

(c)	give the declaration to the building certifier who granted the building permit for the work.

Note for subregulation (4)

See section 69(5) of the Act or regulation 20D(5), as applicable, in relation to protection from prosecution for criminal liability if the declaration is made in good faith.

(5)	In this regulation:

appropriate person means:

(a)	if the building work was partially completed by the responsible builder – the person who is continuing to carry out the building work or is in charge of carrying out the continuing building work; or

(b)	if the building work was fully completed – a person authorised by the corporation in writing to make the declaration under subregulation (4).

20F	Building certifier to give copy of declaration to Director

A building certifier given a declaration made under regulation 20C must, not later than 7 days after receiving the declaration, give a copy of it to the Director.

Penalty:	$1 000.

Part 4	Approvals etc., by other persons or bodies

21	Certificates by other persons

For section 40(a) of the Act, but without limiting that provision, a building certifier may rely on a certificate that work complies with the Act and these regulations given by:

(a)	in relation to the compliance of the design of the building with these Regulations – an architect;

(b)	in relation to the structure of a building – a structural engineer;

(c)	in relation to the mechanical systems of a building – a mechanical engineer;

(d)	in relation to the hydraulic services of a building – a hydraulic engineer;

(e)	in relation to the plumbing, drainage work and on-site sewage disposal system of a building – a plumber or drainer;

(f)	in relation to the design of the plumbing and drainage works and on-site sewage disposal systems – a plumber and drainer (design),

who is a registered building practitioner under the Act.

Part 5	Protection of adjoining property

22	Protection of adjoining property

(1)	Where an excavation or demolition is to be made in proximity to an existing building that building shall be protected to ensure stability.

(2)	Where the foundation of an existing building is of material likely to become unstable as a result of the excavation of adjoining ground additional precautions to the satisfaction of the building certifier shall be taken to ensure stability.

Part 6	Enforcement of building standards

23	Forms of building notice, building order

(1)	A building notice shall be in the form of Form 1 in Schedule 3.

(2)	A building order shall be in the form of Form 2 in Schedule 3.

24	Management and fire evacuation training

The matters prescribed for the purposes of section 116(2) are directions relating to:

(a)	the management and operation of the building; and

(b)	the training with respect to fire evacuation procedures to be undertaken by the occupants.

25	Notification, cancellation or amendment of building order

(1)	A building certifier who makes a building order shall notify the Director within 48 hours of the making of the building order.

(2)	The Director is taken to have refused a request for the amendment or cancellation of a building order if the Director has not decided the request within 10 days after the day on which the request is made.

Part 7	Appeals

26	Enforcement of determination

(1)	An appeal, referral or application to the Building Appeals Board shall be in the approved form and shall set out the grounds of appeal or in the case of a referral or application shall set out the matter the subject of the referral or application.

(2)	A person who makes an appeal, referral or application shall serve a copy of the appeal, referral or application on the other party to the appeal, referral or application, as the case requires, not later than 2 days after lodging the appeal, referral or application at the office of the Building Appeals Board.

(3)	The prescribed court under section 142 of the Act for the purposes of the enforcement of determinations of the Building Appeals Board is the local court.

Part 8	General enforcement provision

27	Recovery of penalties

Proceedings for an offence against the Act are to be dealt with by a court of summary jurisdiction constituted by a magistrate.

28	Orders to be enforced by Local Court

An order referred to in section 152 is to be enforced in the local court.

29	Notice of authorisation

The Minister shall issue a notice of authorisation to each person appointed as an authorised officer under section 162 of the Act and the holder shall produce the notice whenever required to do so by a person in charge of any premises in relation to which the authorised person has exercised or is about to exercise his or her powers under the Act.

Part 9	Siting requirements

30	Interpretation

(1)	In this Part, unless the contrary intention appears, siting includes plot ratio, site coverage and set-back.

(2)	Where the corner of an allotment has been truncated, the area of the allotment shall, for the purposes of this Part, include the area of the truncated part.

31	Application

This Part does not apply to or in relation to the siting of a Class 1 or Class 10 Building in any part of the Territory in relation to which there is in force development provisions, or an interim development control order, under the Planning Act providing for the siting of such buildings.

32	Minimum distances for Class 1 or Class 10 buildings

Subject to these regulations, a Class 1 or Class 10 Building shall be situated on an allotment not less than:

(a)	7.5 m from the street alignment of a primary street;

(b)	where the allotment has a frontage to more than one street – 2.5 m from the street alignment of the secondary street; and

(c)	1.5 m from boundaries other than the street alignments referred to in paragraphs (a) or (b).

33	Exceptions

(1)	A verandah, carport, pergola or other approved building, constructed without external walls, may be constructed less than 7.5 m from the street alignment of a primary street, but no part of the building shall be constructed less than 4.5 m from that street alignment unless approved.

(2)	The requirements of regulation 32(c) do not apply to or in relation to a garden shed, to be constructed on an allotment less than 1.5 m from a side or rear boundary of the allotment, that:

(a)	is not more than 6 m2 in area;

(b)	is not more than 2.3 m in height;

(c)	has no openings in its walls which adjoin a boundary; and

(d)	does not discharge rainwater on an adjacent allotment.

34	Minimum distance for a Class 1 building in Municipality of Palmerston

Regulation 32 does not apply to or in relation to a Class 1 Building in the Municipality of Palmerston, and such a building in the Municipality shall be situated on an allotment not less than:

(a)	7.5 m from the street alignment of a primary street;

(b)	where the allotment has a frontage to more than one street – 2.5 m from the street alignment of the secondary street;

(c)	5 m from the rear boundary of the allotment; and

(d)	3 m from any other boundary.

35	More than one Class 1 building on one allotment

A person shall not construct more than one Class 1 Building on an allotment except where:

(a)	a planning scheme or interim development control order allows the construction; or

(b)	the allotment is not less than 4 ha in area, not more than 2 buildings are constructed and both buildings comply with the Regulations in this Part in relation to siting requirements for separate sites.

36	Plot ratio of Class 1 buildings

(1)	Where a Class 1 Building or a Class 1 Building and one or more Class 10 Buildings are constructed on an allotment the total roofed area shall not exceed 50 per cent of the area of the allotment unless consent to exceed that percentage is conferred by development provisions, or an interim development control order, under the Planning Act.

(2)	For the purposes of calculating the roofed area for the purposes of subregulation (1) the area of a verandah, carport, pergola or other building on the site that has no external walls may be excluded.

Part 9A	Relocation of approved Class 1a Buildings to approved rural land

36A	Definitions

In this Part:

approved rural land means land in one of the following zones, as specified in a planning scheme:

(a)	Agriculture;

(b)	Horticulture;

(c)	Rural;

(d)	Rural Living.

original site, of an approved Class 1a Building, means the land on which the building was originally erected.

post-relocation building work, for an approved Class 1a Building, means any of the following:

(a)	footings and fixture work as defined in regulation 36D(4);

(b)	plumbing connection work as defined in regulation 36E(2);

(c)	electrical installation connection work as defined in regulation 36F(2).

36B	Application of Part

(1)	This Part applies only in relation to:

(a)	an approved Class 1a Building that:

(i)	is to be removed from its current site for relocation on approved rural land; or

(ii)	has been relocated on approved rural land, whether before or after the commencement of this regulation; and

(b)	post-relocation building work.

(2)	This Part does not affect a person's obligation to comply with requirements under another law in force in the Territory that relates to the erection of an approved Class 1a Building on approved rural land (for example, the Planning Act).

36C	Approved Class 1a Building

(1)	A building is an approved Class 1a Building if:

(a)	the building work for the original erection of the building started on or after 1 January 1975; and

(b)	after completion of the building work, the building was a Class 1a Building (or the equivalent class) under the relevant building law; and

(c)	the building was originally erected on, and affixed to, land to which the relevant building law applied; and

(d)	the original site of the building meets the design wind speed requirement for relocation of the building to approved rural land; and

(e)	one of the following (the original building status) applies to the building:

(i)	it was built by or for the Crown in right of the Commonwealth;

(ii)	a permit or certificate allowing occupancy of the building was granted for it under the relevant building law.

(2)	The original site of the building meets the requirement mentioned in subregulation (1)(d) if the design wind speed applicable to the site is the same as, or greater than, the design wind speed applicable to the approved rural land to which it is proposed to relocate the building.

(3)	In this regulation:

design wind speed, in relation to a site, means the design wind speed under these Regulations applicable to the design and construction of buildings that may be approved on the site.

36D	Footings and fixture work

(1)	Before a person starts to carry out footings and fixture work for an approved Class 1a Building that is to be (or has been) relocated to approved rural land, a building permit must be in force for the work.

(2)	The building permit must include the following details:

(a)	the location of the original site of the approved Class 1a Building (for example, the lot number and general location);

(b)	the original building status of the building.

(3)	The footings and fixture work must be carried out by a registered building contractor or an owner-builder.

(4)	In this regulation:

footings and fixture work, for the Class 1a Building, means:

(a)	the construction on the approved rural land of footings for the building; and

(b)	the fixture of the building to the footings.

36E	Plumbing connection work

(1)	Before a person starts to carry out plumbing connection work for an approved Class 1a Building that is to be (or has been) relocated to approved rural land, a building permit must be in force for the work.

(2)	In subregulation (1):

plumbing connection work means work for connecting the approved Class 1a Building to a plumbing and drainage service on the approved rural land.

36F	Electrical installation connection work

(1)	A certificate of compliance, as mentioned in section 69A(1)(c) of the Act, is required for electrical installation connection work for an approved Class 1a Building that is to be (or has been) relocated to approved rural land.

(2)	In subregulation (1):

electrical installation connection work means work, to which section 69 of the Electricity Reform Act applies, for connecting the approved Class 1a Building to an electricity supply for the approved rural land.

36G	Occupancy permit and structural report

(1)	A building certifier must not issue an occupancy permit relating to post-relocation building work for an approved Class 1a Building, as specified in a building permit, unless the application for the occupancy permit is accompanied by a report by a registered certifying engineer (structural):

(a)	made after completion of the footings and fixture work for the building, as defined in regulation 36D(4); and

(b)	stating the approved Class 1a Building has not been structurally compromised during its relocation to the approved rural land.

(2)	An occupancy permit issued in relation to post-relocation building work for an approved Class 1a Building, as specified in a building permit, must include a statement to the effect that the building retains its original building status.

Part 10	Areas liable to flooding

37	Flood prone areas

The parts of the Territory included in Schedule 4 are prescribed as flood prone areas.

38	Flood levels

(1)	Subject to a determination under this regulation, the flood level for a flood prone area is the flood level for a 1 in 100 year flood level.

(2)	The Director may by notice in the Gazette determine the flood level in relation to any flood prone area.

(3)	A notice under subregulation (1) may be amended or revoked by the Director by notice in the Gazette.

(4)	A notice under this regulation may be given in relation to a flood prone area generally or any part of such an area.

39	Requirements in flood prone areas

The following provisions apply in relation to a building constructed in a flood prone area:

(a)	the height of the lowest floor level, or lowest part of the floor level, of a habitable room shall be not less than 300 mm above the flood level for the flood prone area;

(b)	the structural design of the building shall be adequate to withstand flooding and for this purpose special consideration shall be given to:

(i)	the site, size and shape of the building;

(ii)	the effect of buoyancy on the sub-structure of the building; and

(iii)	the stresses that the depth and velocity of water and the impact of water borne debris may have on the structure.

Part 11	Building practitioners

39A	Sub-categories of building practitioner

For section 4A(2)(b) of the Act, the sub-categories specified in the table below are prescribed for the category of building practitioner specified opposite.

|  | Category | Sub-categories |
| --- | --- | --- |
| 1. | building certifier | building certifier (residential) building certifier (unrestricted) |
| 2. | certifying plumber | certifying plumber and drainer certifying plumber and drainer (design) |
| 3. | certifying engineer | certifying engineer (structural) certifying engineer (hydraulic) certifying engineer (mechanical) |
| 4. | building contractor | building contractor residential (restricted) building contractor residential (unrestricted) |

40	Condition for registration: professional indemnity insurance

(1)	For sections 24(1)(c), (2)(c) and (3)(c) and 24F(1)(e), (2)(d) and (3)(d), it is a condition for registration or renewal of registration in a category of building practitioner specified in subregulation (2) that the applicant hold a policy of professional indemnity insurance for the amount determined by the Minister.

(2)	Subregulation (1) applies in relation to all categories of building practitioner other than the following:

(a)	certifying plumber and drainer;

(b)	building contractor.

40A	Condition of registration: HBCF plumber policy

(1)	For sections 24(1)(c), (2)(c) and (3)(c) and 24F(1)(e), (2)(d) and (3)(d), it is a condition for registration or renewal of registration in the category of certifying plumber and drainer that the applicant hold an HBCF plumber policy.

(2)	This regulation expires on the day on which section 61 of the Act is repealed.

Note for subregulation (2)

Section 61 of the Act will be repealed by section 21 of the Building Amendment Act 2004.

(3)	In subregulation (1):

HBCF plumber policy means a policy of insurance that:

(a)	relates to building work carried out by a certifying plumber and drainer; and

(b)	is approved for section 61 of the Act.

40B	Last day for application for renewal of registration

For section 24E(2) of the Act, the prescribed date is the date 2 months before the applicant's registration expires.

Example for regulation 40B

If a building practitioner's registration expires on 31 December 2007, the Practitioners Board may refuse to accept the practitioner's application for renewal if it is made after 31 October 2007. If that happens, the practitioner may apply for registration under section 23 or 24A of the Act.

41	Fees

The fees set out in Schedule 1 are payable in relation to the matters specified in that schedule.

Part 11A	Building contractors

Division 1	Prescribed matters for section 48A of the Act

41A	Building work

(1)	For section 48A(1) of the Act, prescribed building work is work for or in connection with the construction of any of the following:

(a)	a Class 1a building;

(b)	a Class 2 building;

(c)	a Class 10 building attached to a Class 1a or Class 2 building if it is constructed at the same time as the Class 1a or Class 2 building is constructed;

(d)	a retaining wall (whenever constructed) that is not attached to a Class 1a or Class 2 building but on which the integrity of such a building depends.

Examples of Class 10 buildings for subregulation (1)(c)

1.	A garage attached to a Class 1a or Class 2 building.

2.	A retaining wall attached to a Class 1a or Class 2 building.

(2)	However, the prescribed building work does not include work for or in connection with the construction of any of the following:

(a)	renovations or alterations to an existing Class 1a or Class 2 building if there is no increase in the floor area of the building;

(b)	a Class 10 building or verandah attached to a Class 1a or Class 2 building if it is constructed at any time after the Class 1a or Class 2 building was constructed;

(c)	a Class 10 building (whenever constructed) that is not attached to a Class 1a or Class 2 building, other than a retaining wall referred to in subregulation (1)(d).

Examples of renovations and alterations for subregulation (2)(a)

1.	The re-cladding of the roof or walls of a Class 1a or Class 2 building.

2.	The replacement of windows in a Class 1a or Class 2 building.

3.	The construction of new external openings in a Class 1a or Class 2 building.

4.	The enlargement or filling in of existing external openings in a Class 1a or Class 2 building.

Examples of Class 10 buildings for subregulation (2)(c)

1.	A freestanding garage or shed.

2.	A retaining wall that is not attached to a Class 1a or Class 2 building and on which the integrity of a Class 1a or Class 2 building does not depend.

41B	Building contractors for detached houses and attached dwellings and Class 2 buildings of not more than 2 storeys

(1)	For section 48A(1)(a) and (b) of the Act, the following building contractors are prescribed for building work referred to in subregulation (2):

(a)	a building contractor residential (restricted);

(b)	a building contractor residential (unrestricted).

(2)	Subregulation (1) applies in relation to building work that is work for or in connection with the construction of any of the following:

(a)	a Class 1a detached house;

(b)	a Class 1a attached dwelling or Class 2 building of one or 2 storeys;

(c)	a Class 10 building attached to a Class 1a or Class 2 building referred to in paragraph (a) or (b) if it is constructed at the same time as the Class 1a or Class 2 building;

(d)	a retaining wall (whenever constructed) that is not attached to a Class 1a or Class 2 building referred to in paragraph (a) or (b) but on which the integrity of such a building depends.

Examples of Class 10 buildings for subregulation (2)(c)

1.	A garage attached to a Class 1a or Class 2 building.

2.	A retaining wall attached to a Class 1a or Class 2 building.

(3)	However, subregulation (1) does not apply in relation to work for or in connection with the construction of any of the following:

(a)	a Class 10 building or verandah attached to a Class 1a or Class 2 building if it is constructed at any time after the Class 1a or Class 2 building was constructed;

(b)	a Class 10 building (whenever constructed) that is not attached to a Class 1a or Class 2 building, other than a retaining wall referred to in subregulation (2)(d).

Examples of Class 10 buildings for subregulation (3)(b)

1.	A freestanding garage or shed.

2.	A retaining wall that is not attached to a Class 1a or Class 2 building and on which the integrity of a Class 1a or Class 2 building does not depend.

41C	Building contractors for attached dwellings and Class 2 buildings of more than 2 storeys

(1)	For section 48A(1)(a) and (b) of the Act, a prescribed building contractor for building work referred to in subregulation (2) is a building contractor residential (unrestricted).

(2)	Subregulation (1) applies in relation to building work that is work for or in connection with the construction of a Class 1a attached dwelling or Class 2 building of more than 2 storeys.

41D	Supervision by building contractor

For section 48A(1)(b) of the Act, the prescribed building contractor must supervise the person by the level of personal oversight the contractor considers appropriate, having regard to the person's skills and experience.

41E	Minimum value of building work

For section 48A(2)(a) of the Act, the prescribed amount is $12 000.

Division 2	Prescribed matters for section 48B of the Act

41F	Building contractors

For section 48B(1) of the Act, the following building contractors are prescribed:

(a)	a building contractor residential (restricted);

(b)	a building contractor residential (unrestricted).

41G	Building work

(1)	For section 48B(1) of the Act, prescribed building work is work for or in connection with the construction of any of the following:

(a)	a Class 1a detached house;

(b)	a Class 10 building attached to a Class 1a detached house if it is constructed at the same time as the house is constructed;

(c)	a retaining wall (whenever constructed) that is not attached to a Class 1a detached house but on which the integrity of such a house depends.

Examples of Class 10 buildings for subregulation (1)(b)

1.	A garage attached to a detached house.

2.	A retaining wall attached to a detached house.

(2)	However, the prescribed building work does not include work for or in connection with the construction of any of the following:

(a)	renovations or alterations to an existing Class 1a detached house if there is no increase in the floor area of the house;

(b)	a Class 10 building or verandah attached to a Class 1a detached house if it is constructed at any time after the house was constructed;

(c)	a Class 10 building (whenever constructed) that is not attached to a Class 1a detached house, other than a retaining wall referred to in subregulation (1)(c).

Examples of renovations and alterations for subregulation (2)(a)

1	The re-cladding of the roof or walls of a detached house.

2	The replacement of windows in a detached house.

3	The construction of new external openings in a detached house.

4	The enlargement or filling in of existing external openings in a detached house.

Examples of Class 10 buildings for subregulation (2)(c)

1	A freestanding garage or shed.

2	A retaining wall that is not attached to a detached house and on which the integrity of a detached house does not depend.

(3)	In addition, the prescribed building work does not include work for or in connection with:

(a)	the relocation of an approved Class 1a Building to approved rural land; or

(b)	post-relocation building work.

41H	Contents of building contracts

For section 48B(2) of the Act, the contract must:

(a)	be for building work to be carried out on a single project; and

(b)	identify the building contractor; and

(c)	specify the building contractor's registration number; and

(d)	specify the extent and value of the work; and

(e)	if the contract provides for the payment of a deposit – specify the amount of the deposit, which must not be more than 5% of the value of the work; and

(f)	the stages of the work to which progress payments are linked; and

(g)	the amount of each progress payment; and

(h)	set out a process for resolving disputes between the building contractor and the owner of the land.

41J	Minimum value of building work

For section 48B(3)(a) of the Act, the prescribed amount is $12 000.

Division 3	Prescribed matters for section 48C of the Act

41K	Building contractors

For section 48C(2) of the Act, the following building contractors are prescribed:

(a)	a building contractor residential (restricted);

(b)	a building contractor residential (unrestricted).

41L	Building work

(1)	For section 48C(1) of the Act, prescribed building work is work for or in connection with the construction of any of the following:

(a)	a Class 1a detached house;

(b)	a Class 10 building attached to a Class 1a detached house if it is constructed at the same time as the house is constructed;

(c)	a retaining wall (whenever constructed) that is not attached to a Class 1a detached house but on which the integrity of such a house depends.

Examples of Class 10 buildings for subregulation (1)(b)

1.	A garage attached to a detached house.

2.	A retaining wall attached to a detached house.

(2)	However, the prescribed building work does not include work for or in connection with the construction of any of the following:

(a)	renovations or alterations to an existing Class 1a detached house if there is no increase in the floor area of the house;

(b)	a Class 10 building or verandah attached to a Class 1a detached house if it is constructed at any time after the house was constructed;

(c)	a Class 10 building (whenever constructed) that is not attached to a Class 1a detached house, other than a retaining wall referred to in subregulation (1)(c).

Examples of renovations and alterations for subregulation (2)(a)

1.	The re-cladding of the roof or walls of a detached house.

2.	The replacement of windows in a detached house.

3.	The construction of new external openings in a detached house.

4.	The enlargement or filling in of existing external openings in a detached house.

Examples of Class 10 buildings for subregulation (2)(c)

1.	A freestanding garage or shed.

2.	A retaining wall that is not attached to a detached house and on which the integrity of a detached house does not depend.

Part 11B	Owner-builders

Division 1	Prescribed matters for section 48D of the Act

41N	Building work

(1)	For section 48D(1) of the Act, prescribed building work is work for or in connection with the construction of any of the following:

(a)	a Class 1a detached house;

(b)	a Class 10 building attached to a Class 1a detached house if it is constructed at the same time as the house is constructed;

(c)	a retaining wall (whenever constructed) that is not attached to a Class 1a detached house but on which the integrity of such a house depends.

Examples of Class 10 buildings for subregulation (1)(b)

1.	A garage attached to a detached house.

2.	A retaining wall attached to a detached house.

(2)	However, the prescribed building work does not include work for or in connection with the construction of any of the following:

(a)	renovations or alterations to an existing Class 1a detached house if there is no increase in the floor area of the house;

(b)	a Class 10 building or verandah attached to a Class 1a detached house if it is constructed at any time after the house was constructed;

(c)	a Class 10 building (whenever constructed) that is not attached to a Class 1a detached house, other than a retaining wall referred to in subregulation (1)(c).

Examples of renovations and alterations for subregulation (2)(a)

1.	The re-cladding of the roof or walls of a detached house.

2.	The replacement of windows in a detached house.

3.	The construction of new external openings in a detached house.

4.	The enlargement or filling in of existing external openings in a detached house.

Examples of Class 10 buildings for subregulation (2)(c)

1.	A freestanding garage or shed.

2.	A retaining wall that is not attached to a detached house and on which the integrity of a detached house does not depend.

41P	Minimum value of building work

For section 48D(2)(a) of the Act, the prescribed amount is $12 000.

41Q	Exemption for certain building contractors

For section 48D(2)(b) of the Act, the following building contractors are prescribed:

(a)	a building contractor residential (restricted);

(b)	a building contractor residential (unrestricted).

Division 2	Prescribed matters for section 48F of the Act

41R	Period required to elapse

For section 48F(4), the prescribed period is 6 years.

Part 12	Miscellaneous

42	Offences

A person who contravenes any of the provisions of these regulations commits an offence.

Penalty:	$10,000 and in the case of a continuing offence an additional penalty of $1,000 for each day during which the offence continues.

43	Transitional

Where the Practitioners Board is satisfied on application made in accordance with the Act and these regulations that a person:

(a)	is eligible for accreditation as an assistant building surveyor, by the Building Surveyors and Allied Professions Accreditation Board, or has knowledge and experience, that is in the opinion of the Practitioners Board equivalent for such accreditation, or is the holder of a degree related to building construction, awarded by an Australian tertiary education institution, or a qualification that is in the opinion of the Practitioners Board equivalent to such a degree;

(b)	has had three years practical experience in building work in the Northern Territory relating to Class 1 Buildings, Class 10 Buildings and Class 2 Buildings not exceeding 3 residential storeys under the repealed Act; and

(c)	has had knowledge of the statutory requirements relating to building work in the Northern Territory,

the Practitioners Board may permit the person to act as a building certifier for a period of 3 years commencing from the commencement of the Act.

Part 13	Matters for commencement of Building Amendment Act 2004

44	Definitions

In this Part:

amending Act means the Building Amendment Act 2004.

building means a building constructed under a building permit granted before the commencement of Part 4A of the Act.

work means work for which a building permit had been granted before the commencement of Part 4A of the Act.

45	Inspection stages

For section 62(2) of the Act, the inspection stages of work are the stages notified by the building certifier under section 62(1) of the Act as in force immediately before the commencement of section 22 of the amending Act.

46	Exemption of work from sections 63 and 70

(1)	Sections 63 and 70 of the Act do not apply to work.

(2)	However, sections 63 and 70 of the Act, as in force immediately before the commencement (respectively) of sections 23 and 27 of the amending Act, apply to work.

47	Exemption of occupancy permit for building from section 72

(1)	Section 72 of the Act does not apply to an occupancy permit for a building.

(2)	However, section 72 of the Act, as in force immediately before the commencement of section 29 of the amending Act, applies to the permit.

Part 14	Transitional matters for Parts 4A and 4B of Act

48	Registration of building contractors

(1)	Subregulation (2) applies in relation to a building contractor who was registered under section 24B or 24C of the Act after that section commenced but before Part 4A of the Act commenced.

(2)	Despite section 24D of the Act but subject to the remainder of the Act, the registration of a building contractor to whom this Part applies remains in force until the date 2 years after the commencement of Part 4A of the Act.

49	Building permits granted before commencement of Parts 4A and 4B of the Act

Parts 4A and 4B of the Act do not apply in relation to work for which a building permit had been granted before the commencement of the Parts.

Part 15	Transitional matters for inspection stages of residential building work

50	Inspection stages for pre-July 2006 residential building work

Regulation 15A does not apply to:

(a)	residential building work for which a building permit has been granted prior to 3 July 2006; or

(b)	residential building work for which:

(i)	an application for a building permit was made before 1 January 2007; and

(ii)	a building certifier has certified in writing that substantial progress was made in the design of the building prior to 3 July 2006.

Part 16	Transitional matters for Building Legislation Amendment Act 2010

51	Definitions

In this Part:

commencement day means the day on which the Building Legislation Amendment Act 2010 commences.

repealed, in relation to a specified provision, means that provision as in force immediately before the commencement day.

substituted, in relation to a specified provision, means that provision as in force on the commencement day.

52	Declaration made by corporation after commencement day for building work completed before commencement day

(1)	This regulation applies if, before the commencement day:

(a)	prescribed building work to which repealed regulation 20A(1) applied had been completed by a person carrying out building work for a corporation (the responsible builder); and

(b)	the declaration mentioned in that subregulation had not been given to the building certifier under repealed regulation 20A(3).

(2)	The following regulation, as appropriate, applies in relation to the declaration relating to the building work:

(a)	substituted regulation 20B, other than subregulation (5) of that regulation;

(b)	substituted regulation 20C, other than subregulation (4) of that regulation.

(3)	If the responsible builder ceased to act for the corporation after completing the prescribed work, the corporation must follow the procedures specified in substituted regulation 20E as if a prescribed event had occurred.

(4)	For this regulation, a responsible builder had completed prescribed building work if he or she had completely ceased to carry out the building work specified in the building permit granted for the work, regardless of whether the work was fully or partially completed.

(5)	This regulation does not affect the liability of the responsible builder for an offence against repealed regulation 20A(1), (2) or (3) or substituted regulation 20B, 20C or 20D.

Schedule 1	Prescribed fees

regulation 41

| Item | Description | Revenue units |
| --- | --- | --- |
| 1 | In relation to the registration of a building practitioner (other than a building contractor): (a) for an application under section 23 of the Act (b) for registration under section 24 of the Act | 230 690 |
| 2 | In relation to the registration of a building contractor: (a) for an application under section 24A or 24C of the Act (b) for registration under section 24B or 24C of the Act | 230 690 |
| 3 | In relation to the renewal of the registration of a building practitioner: (a) for an application under section 24E of the Act (b) for renewal under section 24F of the Act | 230 690 |
| 4 | For an application for the grant or renewal of an owner-builder certificate under section 48E of the Act | 230 |
| 5 | For carrying out any function by the Director | full reasonable costs of exercising the function |
| 6 | For a search of the register of building permits | 5 |
| 7 | For a search of the register of occupancy permits | 5 |
| 8 | For access to building records maintained by the Director | 5 |
| 9 | For a certified copy of a document under section 166A of the Act | 57 plus the fee for the copy of the document payable under item 10 |
| 10 | Copies of building records maintained by the Director – per A4 page – per A3 drawing – per sheet of drawings larger than A3 | 1 5 5 |
| 11 | For a search of the register of building orders | 5 |
| 12 | For an appeal to the Building Appeals Board: (a) for a Class 1 Building, a Class 2 Building not exceeding 3 residential storeys or a Class 10 Building (b) for any other Building | 115 230 |
| 13 | For an application (other than an application relating to the modification of Part 9 of the Building Regulations in relation to a Class 1 or Class 10 Building) to the Building Appeals Board for modification of the Building Regulations – for each modification: (a) for a Class 1 Building, a Class 2 Building not exceeding 3 residential storeys or a Class 10 Building (b) for any other building | 115 230 |
| 14 | For an application to the Building Appeals Board relating to the modification of Part 9 of the Building Regulations in relation to a Class 1 or Class 10 Building | 57 |
| 15 | For an inspection of a decision of the Building Appeals Board | 11 |
| 16 | For a copy of a decision of the Building Appeals Board – per page | 1 |

Schedule 2

regulation 8

| Column 1 Reporting Authority | Column 2 Building or building work |  | Column 3 Matter to be reported on |
| --- | --- | --- | --- |
| Northern Territory Fire and Rescue Service established by the Fire and Emergency Act | Any building to be classified as a class 2 building that has more than three residential storeys and any building to be classified as a Class 3, 4, 5, 6, 7, 8, or 9 building. Special hazard buildings including: Aircraft Hangars; Cotton Mills; Distilleries; Film and television studios; High Pile storage warehouses (combustible materials); Buildings where the following materials are to be produced, used or stored: Celluloid Goods; Fire works; Fibre glass products; Foam Rubber; Foam Plastics; Chemicals; Paint, varnish; Woodwool. |  | Emergency vehicle areas and clear space around large isolated buildings. Fire fighting equipment. Smoke control. Emergency lighting, exit signs and warning devices. Maintenance and testings of fire suppression equipment. Heating appliances, fire-places, chimneys and flues. Fire and smoke control systems in buildings containing atriums. Smoke control systems for theatres. |
| Chief Health Officer appointed under the Public Health Act | Special Use Buildings including: Food premises; Premises to be used for activities involving Skin Penetration; Mortuaries. |  | Parts H101, H102 and H103 of the Building Code of Australia. Those matters relating to the registration of the premises under relevant Health Legislation. |
|  | Any building: – to be classified as a Class 3 building; – to be classified as a Class 9a building; – to be used a child minding facility; – to be used for the purposes of carrying out Hairdressing. |  |  |
| Work Health Authority continued under the Work Health Administration Act | Any building: – to be used for the purposes of an educational workroom, abrasive blasting chamber, spray painting booth or laboratory; – to be used for the storage of dangerous goods as defined: • in section 3 of the Dangerous Goods Act; or • under Schedule 1 of the Work Health and Safety (National Uniform Legislation) Act; – that is to contain a lift, escalator or moving walkway; – that is constructed of scaffold that is to be used as a spectator stand. |  | Those matters relating to the Work Health and Safety (National Uniform Legislation) Regulations. |
| Power and Water Corporation established by the Power and Water Corporation Act | Any building: – requiring connection to a power, water or sewerage main of the Authority; – requiring an on site electrical substation; – that will encroach on the safety clearances of an overhead power-line as specified; – requiring the installation of trade waste interceptors or discharge of trade waste to the sewer. |  | For consent to connect to a power, water or sewerage main of the Authority; approval of the size and location of an electrical substation; consent to any encroachment. consent to any interceptor or discharge. |
| The relevant consent authority for a planning scheme or interim development control order under the Planning Act |  | Any building subject to compliance with a planning scheme, interim development control order or development permit under the Planning Act | Compliance with the planning scheme, interim development control order or development permit. |
| Agency having responsibility for roads under the Control of Roads Act. |  | Any building: – that will effect a discharge of surface water onto the road; or – requires any new construction or new alteration to existing conditions or facilities within the road reserve. | Consent to: – the discharge; – the construction or alteration within the road reserve |
| Council for the local government area in which the building is situated or, if the building is not situated in a local government area, the Agency having responsibility for roads and stormwater drainage. |  | Any building: – that will effect a discharge of surface water on to the road, public place or adjoining property; or – requiring any new construction or any alteration to existing conditions or facilities within a road reserve. | Consent to: – the discharge; – the construction or alteration within the road reserve. |

Schedule 3

regulation 23

FORM 1

NORTHERN TERRITORY OF AUSTRALIA

BUILDING NOTICE

LOT/PORTION NO.

OWNER/AGENT:

__________________________________________________________

An inspection of the above mentioned land on                                    has revealed the existence of:–

Therefore you are required to:–

Comments (if any)

Further information on this matter is available from:–

Telephone:

…………………...............................

PERSON SIGNING THIS NOTICE

dated this                      day             of                19

______________________________________________________________

FORM 2

regulation 23

NORTHERN TERRITORY OF AUSTRALIA

BUILDING ORDER

LOT/PORTION NO.

OWNER/AGENT:

________________________________________________________

On                 you were issued with a Building Notice, requiring you to carry out the following:

No satisfactory response has been received from you to date.

Therefore, in accordance with sections 112 and 113 of the Building Act, you are required to:

– within 28 days of the service of this order.

Failure to comply with the requirements of this order may result in legal action being taken under section XXXXX of the Building Act.  The Act provides for a penalty for non compliance (See section 128).

Further information regarding this matter is available from:

Telephone:

..........................…………………...

PERSON SIGNING THIS ORDER

Dated this           day of         19

___________________________________________________________

Schedule 4	Flood prone areas

regulation 37

The areas prescribed as flood prone areas under Schedule 1 clause 9 of the Act are the areas included within a black border on the numbered maps specified below held in the office of the Director.

| Adelaide River | Map Adelaide River | No BBADR 001 |
| --- | --- | --- |
| Alice Springs (Greater Area) | Map Alice Springs | No BBASP 001 |
| Borroloola | Map Borroloola | No BBBOR 001 |
| Darwin (Greater Area) | Map Darwin | No BBDAR 001 |
| Katherine | Map Katherine | No BBKAT 001 |
| Namarada | Map Namarada | No BBDAR 002 |
| Timber Creek | Map Timber Creek | No BBTIM 001 |

Schedule 5	Modifications to the National plumbing code

regulation 4

1.	(1)	Clauses 2 to 5 apply to any part of the Territory to which Parts 2 to 14 of the Act apply.

(2)	Clauses 7 and 8 apply only in relation to the parts of the Territory included in the Darwin Water Supply District as declared under the Water Supply and Sewerage Services Act.

2.	Swimming Pool Discharge.  (AS 3500 Part 2 10.9.)

(1)	A domestic swimming pool must not be connected to a sewer main.

(2)	A swimming pool, other than a domestic swimming pool, must not be connected to a sewer main without the approval of the Power and Water Corporation.

3.	Inspection Openings.  (AS 3500 2.4.7.1.)

An inspection opening to surface must be installed in the following circumstances:

(a)	At the connection to the Power and Water Corporation's sewer in accordance with the Corporation's relevant standard drawing or other surround approved by the Corporation; or

(b)	Where a new section of drain is to be connected to an existing drain; or

(c)	As required by Power and Water Corporation.

4.	Overflow Relief Gullies. (AS 3500 Part 2 2.4.6.6.6)

(a)	Height below lowest fixture.  (AS 3500 Part 2 clause 4.6.6.6)

A minimum height of 100 mm shall be maintained between the top of the overflow gully riser and the lowest fixture connected to the drain.

(b)	Height above surrounding ground.  (AS 3500 Part 2 clause 4.6.6.7.)

The minimum height between the top of the overflow gully riser and the surrounding natural ground surface level shall be 150 mm, except where the gully riser is located in a path or a paved area, where it shall be finished at a level so as to prevent the ponding and ingress of water.

5.	Installation of Cisterns.  (AS 3500 Part 1 clause 9.5)

Where a cistern is installed as part of building it shall be a dual flush cistern conforming to AS 1218 or an equivalent cistern approved by the Power and Water Corporation.

6.	Isolating valves.  (AS 3500 Part 1 clause 5.4)

Where the water supply on an allotment serves more than one dwelling from a main line a readily accessible isolating valve shall be installed for each dwelling so that the water supply of any dwelling can be turned off without affecting the water supply to the other dwelling or dwellings being served by the main line.

7.	Pressure Relief and Venting of Water Heaters and Containers.  (AS 3500 Part 4 4.8(c)(ii).)

An expansion control valve is not required to be fitted to an unvented heater.

8.	Domestic Clothes Washing Machine.  (AS 3500 Part 2 clause 11.26.1.)

A clothes washing machine must not discharge into a floor waste gully.

ENDNOTES

1	KEY

Key to abbreviations

amd = amended 				od = order

app = appendix				om = omitted

bl = by-law					pt = Part

ch = Chapter					r = regulation/rule

cl = clause					rem = remainder

div = Division					renum = renumbered

exp = expires/expired				rep = repealed

f = forms					s = section

Gaz = Gazette					sch = Schedule

hdg = heading					sdiv = Subdivision

ins = inserted					SL = Subordinate Legislation

lt = long title					sub = substituted

nc = not commenced

2	LIST OF LEGISLATION

| Building Regulations 1993 (SL No. 26, 1993) |  |
| --- | --- |
| Notified | 1 September 1993 |
| Commenced | 1 September 1993 |
| Amendments of the Building Regulations (SL No. 39, 1993) |  |
| Notified | 10 November 1993 |
| Commenced | 10 November 1993 |
| Amendments of Building Regulations (SL No. 34, 1995) |  |
| Notified | 4 October 1995 |
| Commenced | 4 October 1995 |
| Amendment of Building Regulations (SL No. 26, 1997) |  |
| Notified | 7 January 1998 |
| Commenced | 7 January 1998 |
| Amendments of Building Regulations (SL No. 28, 1998) |  |
| Notified | 8 July 1998 |
| Commenced | 8 July 1998 |
| Amendments of Building Regulations (SL No. 51, 1998) |  |
| Notified | 11 November 1998 |
| Commenced | 11 November 1998 |
| Planning (Consequential Amendments) Act 1999 (Act No. 56, 1999) |  |
| Assent date | 14 December 1999 |
| Commenced | 12 April 2000 (s 2, s 2 Planning Act 1999 (Act No. 55, 1999) and Gaz S15, 12 April 2000) |
| Amendment of Building Regulations (SL No. 15, 2004) |  |
| Notified | 28 April 2004 |
| Commenced | 28 April 2004 |
| Building Amendment Regulations 2005 (SL No. 38, 2005) |  |
| Notified | 19 October 2005 |
| Commenced | 19 October 2005 |
| Building Amendment Regulations (No. 2) 2005 (SL No. 50, 2005) |  |
| Notified | 14 December 2005 |
| Commenced | 14 December 2005 ( r 2, s 2 Building Amendment Act 2004 (Act No. 65, 2004) and Gaz G50, 14 December 2005, p 4) |
| Statute Law Revision Act 2005 (Act No. 44, 2005) |  |
| Assent date | 14 December 2005 |
| Commenced | 14 December 2005 |
| Building Amendment (Transitional Matters) Regulations (No. 1) 2006 (SL No. 16, 2006) |  |
| Notified | 14 June 2006 |
| Commenced | 3 July 2006 (r 3, s 2 Building Amendment Act 2004 (Act No. 65, 2004) and Gaz S15, 3 July 2006, p 1) |
| Building Amendment (Builder Declarations) Regulations 2006 (SL No. 17, 2006) |  |
| Notified | 14 June 2006 |
| Commenced | 3 July 2006 (r 2, s 2 Building Amendment Act 2004 (Act No. 65, 2004) and Gaz S15, 3 July 2006, p 1) |
| Building Amendment (Transitional Matters) Regulations (No. 2) 2006 (SL No. 24, 2006) |  |
| Notified | 12 July 2006 |
| Commenced | 3 July 2006 (r 3, s 2 Building Amendment Act 2004 (Act No. 65, 2004) and Gaz S15, 3 July 2006, p 1) |
| Building Amendment (Exemption from Inspection) Regulations 2007 (SL No. 2, 2007) |  |
| Notified | 7 February 2007 |
| Commenced | 7 February 2007 |
| Law Reform (Work Health) Amendment Act 2007 (Act No. 30, 2007) |  |
| Assent date | 12 December 2007 |
| Commenced | 1 July 2008 (Gaz S29, 25 June 2008) |
| Statute Law Revision Act 2008 (Act No. 6, 2008) |  |
| Assent date | 11 March 2008 |
| Commenced | 11 March 2008 |
| Local Government (Consequential Amendments) Act 2008 (Act No. 28, 2008) |  |
| Assent date | 14 November 2008 |
| Commenced | 1 July 2008 (s 2) |
| Statute Law Revision Act 2009 (Act No. 25, 2009) |  |
| Assent date | 1 September 2009 |
| Commenced | 16 September 2009 (Gaz G37, 16 September 2009, p 3) |
| Fees and Charges Amendment Regulations 2009 (SL No. 34, 2009) |  |
| Notified | 14 December 2009 |
| Commenced | 1 January 2010 (r 2) |
| Building Legislation Amendment Act 2010 (Act No. 22, 2010) |  |
| Assent date | 30 June 2010 |
| Commenced | 25 August 2010 (Gaz G34, 25 August 2010, p 7) |
| Building Amendment (Rural Relocation) Regulations 2011 (SL No. 46, 2011) |  |
| Notified | 2 November 2011 |
| Commenced | 2 November 2011 |
| Work Health and Safety (National Uniform Legislation) Implementation Act 2011 (Act No. 38, 2011) |  |
| Assent date | 14 December 2011 |
| Commenced | 1 January 2012 (Gaz S79, 30 December 2011) |

3	LIST OF AMENDMENTS

r 2	amd No. 34, 1995, r 2; No. 26, 1997; No. 15, 2004; No. 50, 2005, r 4; Act No. 22, 2010, s 16; No. 46, 2011, r 3

r 3	amd No. 34, 1995, r 2; No. 46, 2011, r 4

rr 3A – 3B	ins No. 46, 2011, r 5

r 4	amd No. 51, 1998, r 1

r 5	amd No. 34, 1995, r 2; No. 38, 2005, r 3; Act No. 44, 2005, s 35

r 10	amd No. 34, 1995, r 2

r 11	amd No. 39, 1993, r 2; Act No. 56, 1999, s 3(2)

r 12	amd No. 34, 1995, r 2

r 13	amd Act No. 22, 2010, s 17

r 14	amd No. 34, 1995, r 2

pt 3 hdg	sub No. 50, 2005, r 5

pt 3

div 1 hdg	ins No. 50, 2005, r 5

r 15A	ins No. 50, 2005, r 5

pt 3

div 2 hdg	ins No. 50, 2005, r 6

r 16A	ins No. 50, 2005, r 6

pt 3

div 3 hdg	ins No. 50, 2005, r 7

r 18	amd No. 34, 1995, r 2

sub No. 50, 2005, r 8

rr 18A – 18C	ins No. 50, 2005, r 8

pt 3

div 4 hdg	ins No. 17, 2006, r 4

rr 20A – 20B	ins No. 17, 2006, r 4

sub Act No. 22, 2010, s 18

rr 20C – 20F	ins Act No. 22, 2010, s 18

r 21	amd Act No. 22, 2010, s 21

r 24	amd No. 34, 1995, r 2

r 26	amd No. 34, 1995, r 2

rr 28 – 29	amd No. 34, 1995, r 2

r 31	amd No. 34, 1995, r 2; Act No. 56, 1999, s 3(2)

r 35	amd No. 34, 1995, r 2; Act No. 6, 2008, s 4

r 36	amd No. 34, 1995, r 2; Act No. 56, 1999, s 3(2)

pt 9A hdg	ins No. 46, 2011, r 6

rr 36A – 36G	ins No. 46, 2011, r 6

pt 11 hdg	amd No. 28, 1998, r 1

r 39A	ins No. 28, 1998, r 2

sub No. 50, 2005, r 9

r 39B	ins No. 28, 1998, r 2

rep No. 50, 2005, r 9

r 40	sub No. 50, 2005, r 10

rr 40A – 40B	ins No. 50, 2005, r 10

pt 11A hdg	ins No. 50, 2005, r 11

pt 11A

div 1 hdg	ins No. 50, 2005, r 11

rr 41A – 41E	ins No. 50, 2005, r 11

pt 11A

div 2 hdg	ins No. 50, 2005, r 11

r 41F	ins No. 50, 2005, r 11

r 41G	ins No. 50, 2005, r 11

amd Act No. 22, 2010, s 19; No. 46, 2011, r 7

rr 41H – 41J	ins No. 50, 2005, r 11

pt 11A

div 3 hdg	ins No. 50, 2005, r 11

r 41K	ins No. 50, 2005, r 11

amd Act No. 22, 2010, s 21

r 41L	ins No. 50, 2005, r 11

r 41M	ins No. 50, 2005, r 11

om Act No. 22, 2010, s 21

pt 11B hdg	ins No. 50, 2005, r 12

pt 11B

div 1 hdg	ins No. 50, 2005, r 12

rr 41N – 41Q	ins No. 50, 2005, r 12

pt 11B

div 2 hdg	ins No. 50, 2005, r 12

r 41R	ins No. 50, 2005, r 12

pt 12 hdg	ins No. 28, 1998, r 3

pt 13 hdg	ins No. 16, 2006, r 4

rr 44 – 47	ins No. 16, 2006, r 4

pt 14 hdg	ins No. 24, 2006, r 4

rr 48 – 49	ins No. 24, 2006, r 4

pt 15 hdg	ins No. 2, 2007, r 3

r 50	ins No. 2, 2007, r 3

pt 16 hdg	ins Act No. 22, 2010, s 20

rr 51 – 52	ins Act No. 22, 2010, s 20

sch 1 hdg	sub No. 50, 2005, r 13

sch 1	amd No. 39, 1993, r 3; No. 50, 2005, r 13

sub No. 34, 2009, r 13

sch 2	amd No. 34, 1995, r 1; No. 51, 1998, r 2; Act No. 44, 2005, s 35; Act No. 6, 2008, s 4; Act No. 30, 2007, s 59; Act No. 28, 2008, s 4; Act No. 25, 2009, s 11; Act No. 38, 2011, s 31

sch 3	amd No. 34, 1995, r 2

sch 4	amd No. 34, 1995, r 2; Act No. 22, 2010, s 21

sch 5	amd Act No. 44, 2005, s 35